Updating the MX44VW

The firmware update process MUST be carried out with your PC’s RJ45 port connected DIRECTLY to the TCP/IP RJ45 port of the Matrix. This update will fail if used in conjunction with any network infrastructure (switch, router, control system etc…).

- Open Firmware upgrading w/ VC & EDID to do the .FW update first.

- Connect to unit and red light will go green

- Select Upgrade Firmware in lower Tab.

- Click Open Firmware File.

- Change the File type to *FW and the latest firmware will show in the folder then choose the folder and press “Open” to accept the file.

- Select the MAIN file ONLY, then press the Upgrade button to start the process

- Once it has finished upgrading you will see a “Firmware Upgrade Successfully message”.

How to update the .Bin file.

1.Go to Network settings and change Ip Address to 192.168.0.210

- Disable the Wifi on computer

- Open Webserver Module Firmware Upgrade Tool Ver2.0 (not 1.0)

2.Open and select the File and press start.

3. Power cycle unit the unit.

4. It will start Updating then once it reaches [9] it will be nearly finished.

5. It will say “Finished” when done.

6. Power cycle the unit by unplugging the power cord for 5 seconds then repowering it.

7. Clear the cache on the browser.