## How to enable or disable ARC and CEC on RX70CS?

✅ What RX70CS Firmware do I need?

The RX70CS has different hardware versions based on when the unit was manufactured. You can identify the hardware version from the 4th character in the serial number, which may be one of the following:

- Version A

- Version B

- Version C

- Version D

Different firmware versions are required depending on the hardware version:

| Hardware Version | Required Firmware Version |
| --- | --- |
| A | V1.3.0a or higher |
| B | V2.3.0a or higher |
| C & D | Already include support in production firmware |

It is critical to load the correct firmware version for your specific hardware. Please refer to the firmware changelog for detailed changes and compatibility.

📟 How to Enable or Disable ARC / CEC

You can use API commands via RS232 to enable or disable CEC and ARC on the RX70CS.

1️⃣ Connect to the RX70CS

You’ll need two cables:

- Serial Cable (DB9 to 3-pin Phoenix Connector) – included with the Blustream HDBT matrix.

- USB to DB9M RS-232 Converter – must be sourced separately

  - Example: Jaycar USB to DB9M RS-232 Converter

Steps:

- Connect your PC to the RX70CS using the RS232 interface.

- Note the COM port number that appears in your PC’s Device Manager.

- Set the DIP switch on the front panel of RX70CS to "UPGRADE".

- Power on the RX70CS.

2️⃣ Launch a UART Communication Tool

You can use any UART terminal software such as:

- Hercules

- UARTAssist

- SSCOM

- PuTTY

Settings:

- Baud Rate: 115200

- Data Bits: 8

- Stop Bits: 1

- Parity: None

- Flow Control: None

- Open the correct COM port.

3️⃣ Send the API Commands

Type the following commands and send them with <CR><LF> (Carriage Return + Line Feed) at the end.

⚠️ Commands are case-sensitive.

| Command | Function |
| --- | --- |
| CEC ENABLE | Enables CEC communication between RX70CS and TV |
| CEC DISABLE | Disables CEC communication |
| ARC ENABLE | Enables ARC communication |
| ARC DISABLE | Disables ARC communication |

🔁 After Applying Commands

- Once the desired settings are configured, you need switch the DIP switch back to "NORMAL" and reboot the RX70CS.