# RX70CS Firmware Upgrade Instructions

Follow the steps below to update the firmware using the RS232 interface:

1. Connect to the RX70CS

- Use an RS232 cable to connect your PC to the RS232 port on the RX70CS. You will need two cables:

  - Serial Cable (DB9 to 3-pin Phoenix Connector):
This cable is included with the Blustream HDBT matrix.

  - USB to DB9M RS-232 Converter:
This must be prepared separately. Example product:
Jaycar USB to DB9M RS-232 Converter

- Note the COM port number that appears in your PC’s device manager.

- Switch the DIP switch on the RX70CS front panel to “UPGRADE”.

- Power on the RX70CS.

2. Launch the Upgrade Tool

- Open the PortUpgrade software on your PC.

3. Configure Communication Settings

- Select the correct COM port (corresponding to your RS232 cable).

- Set the baud rate to 115200.

- Click “connect” and the button will become green when connected successfully.

4. Load the Firmware File

- Click ‘browser’ to browse and select the firmware file for the RX70CS.

5. Start the Upgrade Process

- Click ‘Open’ to load the firmware.

- Click ‘ForceUpgrade’ to begin the firmware update.

6. Complete the Process

- When the upgrade is successful, you’ll see “Upgrade success!”.

- Switch the DIP switch back to “NORMAL”.

- Reboot the RX70CS to complete the firmware upgrade.