## Firmware Update Instructions for SW21AB-V2

What You Need

- A Standard USB to Micro USB cable

- A Windows PC or laptop

- The software: Firmware upgrading with VC & EDID control tool - ver4.50.exe

- The correct firmware file for your SW21AB-V2

Steps to Update the Firmware

- Connect the Hardware

  - Use a Standard USB to Micro USB cable to connect your laptop/PC to the Micro USB port on the SW21AB-V2.

  - Power ON the SW21AB-V2 unit.

- Open the Firmware Tool

  - Run Firmware upgrading with VC & EDID control tool - ver4.50.exe on your PC.

- Select COM Port

  - Go to the General tab in the tool.

  - Under Matrix Interface Method, click COM Port.

  - Select the correct COM port from the list.

  - Set the Baud rate to 57600.

- Connect to Device

  - Click the Connect button.

  - If successful, the red indicator will turn green.

- Load Firmware File

  - Go to the Upgrade Firmware tab.

  - Click the file icon to browse.

  - Select the correct firmware file, then click Open.

- Start the Firmware Upgrade

  - Click the Upgrade button to begin the firmware update process.

  - The progress bar will display the upgrade status.

- Complete

  - Wait for the message:
"Firmware upgrade successfully!"

  - You can now safely disconnect the device.